Marble Falls Formation Analysis & Investment Thesis

- Natural fracture systems dominate production performance

  - Fractures oriented SW-NE and N-S directions with 400ft spacing cyclicity

  - Horizontal wells drilled E-W or N-S perform better than N-S oriented wells

  - Core samples show 3mm apertures - significant fracture width (“Grand Canyon” for gas molecules)

  - BARS log (Borehole Acoustic Resonance Survey) identifies fracture swarms as dark areas

- Technical completion strategy

  - Target fracture intersections rather than matrix porosity

  - Can drill open hole and rely on natural fractures vs expensive fracking

  - Mile-long laterals sufficient (not 3-mile like shale plays)

  - Porosity interpretation challenges: much displayed porosity false due to wrong matrix density

- Investment rationale

  - Abundant natural fractures + matrix storage combination

  - Proven by EOG’s mud losses and 60-foot flare blowout incident

  - Best wells in high porosity fairway (Palo Pinto/Jack County corridor)

- Operational requirements & risks

  - Water management critical: need 500-1000 barrels/day disposal capacity

  - Dewatering process similar to coal bed methane - “move entire ocean of water”

  - Permitting challenges post-2011 due to earthquake concerns

  - Can repurpose old Strawn wells as disposal (limited to 250 barrels/day)

Successful Marble Falls Case Studies

- Hostel Ranch B7 well economics

  - Made only water initially, required persistence through extended dewatering

  - Peak: ~1M cubic feet gas/day, 150 barrels oil/day after 6 months

  - Located at structural low - dewatered regional fracture network

  - Success enabled nearby Hostel B12H (250 BOD, 4M cubic feet/day)

- Munson 1H operational lessons

  - Peak: 1.3-1.4M cubic feet gas/day from proper pumping unit

  - Previous operator failed with ESP due to insufficient fluid rates

  - Gradual production increase 2012-2014 from conventional beam pump

  - Demonstrates reservoir potential with correct artificial lift

Current Marble Falls Operations

- Basher 2H completion results

  - Just completed fracturing with modified design based on First Prize learnings

  - Switched to 100 mesh proppant, revised ramp schedule, 40/70 tail-in

  - Zach Fellows observed significant fracture complexity during treatment

  - Flowback scheduled this week - targeting high porosity zone

- Infrastructure advantages

  - Access to EOG gathering system: 12" steel gas, 8" poly water lines

  - Existing SWD well available for permitting

  - First Prize 1H making manageable 50 barrels water/day from conglomerate

- Development strategy

  - Chasing Marble Falls between existing well locations

  - Focus on structural highs after dewatering lower areas

  - Horizontal development in proven fairway corridor

Pregnant Shale Investment Opportunity

- Proven analog performance

  - Two vertical wells: >750 million cubic feet cumulative production

  - Initial rates >600 MCF/day from acid stimulation only

  - Maze well (Mustgo Oil) and second operator both used MCA acid

- Technical completion requirements

  - Critical: use MCA (monochloroacetic acid) not HCL

  - Formation contains chloride + iron bearing minerals - HCL damages reservoir

  - 80% MCA concentration maximum due to potency/safety concerns

- Current development status

  - Ray well completion scheduled Thursday targeting porosity streak

  - Porosity zone correlates across field: Singleton through Harangue units

  - First pregnant shale shot yesterday already building pressure

- Regional expansion potential

  - Wedge-oriented deposits extend into Parker County with better development

  - 4100 feet pay demonstrated in single lateral

  - Southern resistivity indicates wet formation (10-20 ohms, 10-11% porosity)

  - Uphole from Barnett/Marble Falls - can develop while managing lower zones

- Investment thesis for coalition development

  - 2+ counties of undeveloped potential in wedge fairways

  - Low water production vs high gas rates

  - Can build portfolio combining PDP recompletions with new drilling